SUBGRIDS Extension to the RUN File Format

Noise models in the NOISEMAP family have traditionally supported noise computations only on a single, regular, rectangular, 2-dimensional grid of points. Therefore, the grid point spacing of the entire grid is determined by the maximum resolution needed in any region of the computational domain. This results in a grid with a very large number of points.

This has been tolerable in the past, when most noise calculations were done with NMAP, which runs very quickly. However, the increasing use of AAM, which due to its more accurate simulation-based design runs much slower than NMAP, has caused a crisis in noise model run times. To address this problem, support was added to AAM (beginning with version 3.1.0) for multi-fidelity grids: i.e., grids whose point spacing varies with location. The grid point spacing can be varied depending on the resolution required in any given region.

This multi-fidelity grid support is built upon the concept of nested subgrids, support for which has long been built into NMPlot and the NMGF grid file format, and which is utilized by INM.

Overview of Nested Subgrids

A description of nested subgrids begins with the primary grid, a regular, rectangular grid of points that covers the entire computational domain. The primary grid is equivalent to what the NOISEMAP community has long referred to as simply "the noise grid", and is defined in exactly the same way. The only difference is that, since we can now increase the grid resolution in selected regions, the primary grid can have a relatively low resolution.

Once the primary grid is defined, one or more subgrids can be defined. Each subgrid has an associated parent grid, which is either the primary grid or another subgrid.  A subgrid has twice the resolution (i.e., the distance between adjacent grid points is one half that) of its parent grid, and covers a subset of the area that the parent covers. By repeatedly nesting subgrids, the noise field in a particular area of interest can be defined to any resolution desired.

Conceptually, a nested subgrid is created by taking a rectangular area of the parent grid and adding new grid points exactly half-way between existing parent grid points, so that a new rectangular grid is formed with twice the resolution.  The relationship between a parent grid and a subgrid is illustrated in the following figure.

Each subgrid is defined by the following parameters:

- NAME, a unique identifying name for the subgrid.

- PARENT, used to identify the nested subgrid's parent grid.  It must match the NAME parameter of a previously defined subgrid, or else refer to the primary grid.

- Iparent and Jparent, integer indices of the grid point in the parent grid that serves as the subgrid's origin (i.e., the subgrid's [1, 1] grid point).

- NI and NJ, the number of subgrid points in the i- and j-axis (i.e., the east-west and north-south) directions.

Note that some of the subgrid points also appear in the parent grid.  In the example provided by the above figure, subgrid point [1, 1] is the same as parent grid point [Iparent, Jparent].  To conserve space in NMGF grid files containing subgrids, those points that appear in a parent's grid are not repeated in a subgrid's SUBG section. Also, when performing noise computations for a subgrid, AAM doesn't need to perform calculations at these shared points, since the noise at these points has already been calculated.

The following conditions always hold, and are byproducts of the way in which nested subgrids are defined:

- NI and NJ must always be odd numbers  3.

- A subgrid's parameters must be defined so that it is completely contained within the geographic area covered by its parent grid.

Note: It is recommended that readers of this document review the portion of the NMGF grid file format document that describes the SUBG section (specifically, page 47 in the Nmgfstd.docx document). This document is distributed with NOISEMAP, and can be found in the Noisemap\NMPlot\NMGF directory.

Overview of Subgrid Support in BaseOps, AAM, and NMPlot

BaseOps has been modified such that, when editing cases that use the AAM noise model, subgrids can be defined in the Grid section of the BaseOps user interface.

BaseOps has always passed the primary noise grid's parameters to AAM in the COMPUTEGRD section of the AAM RUN file. If subgrids have been specified, the parameters of each subgrid are also included in the RUN file. The run file format has been extended with a new SUBGRIDS section. Its format is described below.

When run, AAM will read the RUN file, extract the parameters of the subgrids from the SUBGRIDS section, and perform the necessary noise computations. AAM will then write the subgrids' noise data into the GRD file using NMGF SUBG sections (as described in the NMGF file format documentation).

NMPlot can then be used to produce contour plots of the noise data in the GRD file. No modifications to NMPlot were necessary, since it already supported GRD files with SUBG sections.

Format of SUBGRIDS section in RUN Files

Subgrid information is stored in a RUN file inside a SUBGRIDS section, an extension to the original RUN file format. The SUBGRIDS section first lists the number of subgrids, followed by the parameters of each subgrid, one per line. The SUBGRIDS section, if it exists, must immediately follow the COMPUTEGRD section in the RUN file.

In the NMGF grid file format, subgrids can be named with arbitrary text. In the SUBGRIDS section of a RUN file, subgrid names must be integer id numbers. Furthermore, these id numbers must be consecutive and begin at one (i.e., the subgrids will have the id numbers 1, 2, 3...). This constraint is intended to ease AAM's implementation of subgrid support by allowing the subgrid parameters can be stored in simple arrays indexed by the subgrid id numbers.

The SUBGRIDS section format is defined in the following table.

| Line | Columns | Format | Description |
| --- | --- | --- | --- |
| 1 | 1-8 | A8 | The literal text SUBGRIDS, a keyword introducing the SUBGRIDS section |
|  | 11-20 | I10 | Number of subgrids. |
| >= 2 | 1-10 | I10 | The id number of the subgrid. Subgrids must be numbered sequentially, starting at 1. Note that since each subgrid's id number is implied by its order in the SUBGRIDS section, these id numbers don't strictly need to be listed. However, they are required as an aid for humans reading a RUN file. |
|  | 11-20 | I10 | The id number of the subgrid's parent. It must be the id number of a previously-defined subgrid, or else 0, in which case the primary grid is the parent. If the parent is a subgrid, that subgrid must have already been listed in the SUBGRIDS section. This imparts an ordering on how subgrids are listed — the parent must always be listed before its children. |
|  | 21-30 | I10 | The subgrid's Iparent parameter, as defined previously in this document. |
|  | 31-40 | I10 | The subgrid's Jparent parameter, as defined previously in this document. |
|  | 41-50 | I10 | The subgrid's NI parameter, as defined previously in this document. |
|  | 51-60 | I10 | The subgrid's NJ parameter, as defined previously in this document. |

Here's a simple example of a SUBGRIDS section in a RUN file. The COMPUTEGRD section is included for reference.

COMPUTEGRD             noise.grd             DNL

-50000    -50000     50000     50000

1000      1000        0W

SUBGRIDS           2

1         0        12        15        21        35

2         1         5        10         7         7

In this example, there are two subgrids. The first subgrid is named 1, has as its parent the primary grid (remember that the primary grid is implicitly named 0), has its origin at grid point [12,15] in the primary grid, and has the dimensions of 21 and 35 grid points in the i-axis and j-axis directions.

The second subgrid is named 2, has as its parent the subgrid named 1, has its origin at grid point [5,10] in the parent subgrid, and has the dimensions of 7 and 7 grid points in the i-axis and j-axis directions.